# FOSSEE Winter Internship 2025

NAME: DANDUGULA MADHU

Branch: MSc Geo Informatics (1st year)-Post Graduation

Graduation: Btech-CSe_IoT(2021-2025)

Task 1: Sensor–Actuator Integration (Arduino)

Goal:
 Build a simple real-time system that monitors an analog sensor and triggers an actuator based on a threshold.

1. Introduction

A real-time temperature monitoring system is essential in many applications such as home automation, industrial safety, and environmental monitoring. This project uses an Arduino Uno, an analog temperature sensor (LM35/TMP36), two LEDs, and a buzzer to detect temperature levels and trigger visual and audio alerts based on a predefined threshold.

The system continuously reads analog temperature values and responds instantly, making it suitable for real-time embedded control.

2. Objective

- To design and implement a real-time temperature monitoring system.

- To trigger a hot indicator (LED + buzzer) when temperature crosses a threshold.

- To trigger a cool indicator (LED) when temperature is below or equal to the threshold.

- To understand interfacing analog sensors and actuators with microcontrollers.

3. Components Used

| Component | Quantity | Description |
| --- | --- | --- |
| Arduino Uno | 1 | Microcontroller board for processing |
| LM35/TMP36 Temperature Sensor | 1 | Provides analog temperature output |
| LED (Red) | 1 | Indicates temperature above threshold |
| LED (Green/Blue) | 1 | Indicates normal temperature |
| Buzzer | 1 | Audible warning on high temperature |
| 220Ω Resistors | 2 | Current limiting for LEDs |
| Connecting Wires | — | Circuit connections |
| USB Cable | 1 | Power and programming |

4. Circuit Diagram (Description)

Sensor (LM35/TMP36) Connections

- Left pin → 5V

- Middle pin → A0

- Right pin → GND

LED Connections

Hot LED (Red):

- Arduino Pin 9 → 220Ω Resistor → LED Anode (+)

- LED Cathode (−) → GND

Cool LED (Green/Blue):

- Arduino Pin 8 → 220Ω Resistor → LED Anode (+)

- LED Cathode (−) → GND

Buzzer

- Pin 10 → Buzzer (+)

- Buzzer (−) → GND

5. Working Principle

- The LM35/TMP36 produces an analog voltage proportional to temperature.

- Arduino reads this analog value on A0 and converts it into temperature (°C).

- The system compares the measured temperature with a predefined threshold value.

- If temperature exceeds the threshold:

  - Hot LED turns ON

  - Buzzer turns ON

  - Cool LED remains OFF

- If temperature is ≤ threshold:

  - Cool LED turns ON

  - Hot LED and buzzer remain OFF

The loop runs continuously, enabling real-time response.

6. Arduino Code

const int sensorPin = A0;

const int coolLed = 8;

const int hotLed = 9;

const int buzzer = 10;

float threshold = 35.0; // Temperature threshold in °C

void setup() {

pinMode(coolLed, OUTPUT);

pinMode(hotLed, OUTPUT);

pinMode(buzzer, OUTPUT);

Serial.begin(9600);

}

void loop() {

int sensorValue = analogRead(sensorPin);

float voltage = sensorValue * (5.0 / 1023.0);

float temperature = voltage * 100.0;  // LM35/TMP36 calculation

Serial.print("Temperature: ");

Serial.println(temperature);

if (temperature > threshold) {

digitalWrite(hotLed, HIGH);

digitalWrite(buzzer, HIGH);

digitalWrite(coolLed, LOW);

}

else {

digitalWrite(coolLed, HIGH);

digitalWrite(hotLed, LOW);

digitalWrite(buzzer, LOW);

}

delay(200);

}

7. Applications

- Industrial temperature monitoring

- Server room temperature alarms

- Smart home systems

- Greenhouse temperature control

- Safety alert systems

8. Advantages

- Low-cost and easy to build

- Real-time temperature detection

- Audible and visual alerts

- Low power consumption

- Scalable for multiple sensors

9. Conclusion

This project successfully demonstrates a real-time temperature monitoring system using Arduino. By utilizing a simple analog sensor and basic actuators, the system provides immediate feedback when temperature crosses a critical limit. It serves as a practical example of embedded system design, sensor interfacing, and actuator control.